Материалы для самостоятельной работы обучающихся

Задание для самостоятельной работы 1

### Dynamo

The dynamo was the first electrical generator capable of delivering power for industry. The dynamo uses electromagnetic principles to convert mechanical rotation into an alternating electric current. A dynamo machine consists of a stationary structure which generates a strong magnetic field, and a set of rotating windings which turn within that field. On small machines the magnetic field may be provided by a permanent magnet; larger machines have the magnetic field created by electromagnets.

The first dynamo based on Faraday's principles was built in 1832 by Hippolyte Pixii, a French instrument maker. It used a permanent magnet which was rotated by a crank. The spinning magnet was positioned so that its north and south poles passed by a piece of iron wrapped with wire. Pixii found that the spinning magnet produced a pulse of current in the wire each time a pole passed the coil. Furthermore, the north and south poles of the magnet induced currents in opposite directions. By adding a commutator, Pixii was able to convert the alternating current to direct current.

Unlike the Faraday disc, many turns of wire connected in series can be used in the moving windings of a dynamo. This allows the terminal voltage of the machine to be higher than a disc can produce, so that electrical energy can be delivered at a convenient voltage. The relationship between mechanical rotation and electric current in a dynamo is reversible; the principles of the electric motor were discovered when it was found that one dynamo could cause a second interconnected dynamo to rotate if current was fed through it.

The construction of a dynamo is similar to that of an electric motor, and all common types of dynamos could work as motors.

### Jedlik's dynamo

In 1827, Anyos Jedlik started experimenting with electromagnetic rotating devices which he called electromagnetic self-rotors. In the prototype of the single-pole electric starter (finished between 1852 and 1854) both the stationary and the revolving parts were electromagnetic. He formulated the concept of the dynamo at least 6 years before Siemens and Wheatstone. In essence the concept is that instead of permanent magnets, two electromagnets opposite to each other induce the magnetic field around the rotor.

### Gramme dynamo

Both of these designs suffered from a similar problem: they induced "spikes" of current followed by none at all. Antonio Pacinotti, an Italian scientist, fixed this by replacing the spinning coil with a toroidal one, which he created by wrapping an iron ring. This meant that some part of the coil was continually passing by the magnets, smoothing out the current. Zénobe Gramme reinvented this design a few years later when designing the first commercial power plants, which operated in Paris in the 1870s. His design is now known as the Gramme dynamo. Various versions and improvements have been made since then, but the basic concept of a spinning endless loop of wire remains at the heart of all modern dynamos.

### Exercise 8. Choose the correct form.

1. Development of nuclear power plants for civil use (began, has began, will begin) in the mid 1950s.  2. Heat losses should (reduce, be reduced, have reduced) constantly and effectively. 3. Electric charges (are acted, had been acted, will be acted) upon by forces when they move in the magnetic field. 4. Electricity generated by the conversion of light or mechanical energy (is used, used, will use) mainly for portable power sources. 5. Copper (was used, used, has been used) as a conductor since the beginning of the industry. 6. Now we (are using, use, are used) traditional power sources, that is, oil, gas, natural gas, coal and water power. 7. The world power capacity (is doubling, was being doubled, will be doubled) about per decade. 8. In some projects water (will be heated, heated, heats) by the warmth of mountains at a depth of four-five km. 9. The temperature (has been maintained, has maintained, will be maintained) at the point of 20 degrees since the beginning of the experiments. 10. The experiments (were being conducted, will be conducted, conducted) in some laboratories when the new term began.

Exercise 9. Paraphrase the following sentences using passive construction.

1. We often meet with difficulties in solving the problems of automatic control. 2. I will have made all the experiments necessary for the completion of the research by the end of the year. 3. The scientists of this research institute showed the engineer a new device which they had constructed. 4. They often referred to these measurements at the conference. 5. These scientists have reviewed the role of texture on properties of titanium alloys. 6. We have described the factors which affects the formation of these constituents in detail previously. 7. They are installing a totally new safety system.

### Exercise 10. Think of questions to the following sentences.

### Example:    A steam power machine was invented by J.Watt.

### What machine was invented by J.Watt?

### Who invented the steam power machine?

### 1. Industrial enterprises are commonly built in the areas being rich in natural resources.

### __________________________________________________________________

### 2. The disadvantage of power plants utilizing nuclear fuel is their radiation.

### __________________________________________________________________

### 3. In most areas of the world, mercury is used to extract gold from ore.

### __________________________________________________________________

### 4. The principal components of power plant are the nuclear reactor, the heat exchanger and steam turbine.

### __________________________________________________________________

### 6. The cooling water is drawn from a source and passed through the condenser.

### __________________________________________________________________

### 7. Britain’s first atomic station has begun to generate power in 1956.

### ___________________________________________________________________

### 8. Lomonosov and Franklin happened to make their experiments in the field of atmospheric electricity at about the same time.

### __________________________________________________________________

### Exercise 11. Translate the following sentences paying attention to the voice and the tense form of the predicates.

### 1. Electrical loads or consumers in power installations are the various arrangements and installations that are used to transform electric power into mechanical, heat and chemical power, or energy. 2. In order to meet demands for power during the day, utilities have been turning to other forms of hydroelectric systems. 3. A gas-cooled reactor is nuclear reactor in which   the coolant is a gas. 4. Pierre Curie examined properties of crystals which led him to the discovery of piezoelectric properties. 5.The energy industry is undergoing considerable development. 6. What do substation power supply circuits depend on? 7. The lowest temperature theoretically possible is called the absolute zero. 8. The amount of heat depends on the current and the time it flows. 9. A few pounds of uranium can supply a medium-sized town with power it needs for a year. 10. In the electric generating field as well as in the transmission and storage-technology areas considerable attention is considered on the use of superconducting conductors because of their ability of losing their resistance under certain conditions, namely at temperatures close to absolute zero.

Exercise 12. Read the following supplementary text and translate it.

Задание для самостоятельной работы 2 NISSAN DIESEL

Transformers

A transformer is an electrical device that transfers energy from one circuit to another by magnetic coupling with no moving parts. A transformer comprises two or more coupled windings, or a single tapped winding and, in most cases, a magnetic core to concentrate magnetic flux. A changing current in one winding creates a time-varying magnetic flux in the core, which induces a voltage in the other windings.

The transformer is one of the simplest of electrical devices, yet transformer designs and materials continue to be improved. Transformers are essential for high voltage power transmission, which makes long distance transmission economically practical. This advantage was the principal factor in the selection of alternating current power transmission in the "War of Currents" in the late 1880s.

Transformers come in a range of sizes from a thumbnail-sized coupling transformer hidden inside a stage microphone to huge gigawatt units used to interconnect large portions of national power grids. All operate with the same basic principles and with many similarities in their parts.

The transformer may be considered as a simple two-wheel 'gearbox' for electrical voltage and current. The primary winding is analogous to the input shaft and the secondary winding to the output shaft. In this analogy, current is equivalent to shaft speed, voltage to shaft torque. In a gearbox, mechanical power (torque multiplied by speed) is constant (neglecting losses) and is equivalent to electrical power (voltage multiplied by current) which is also constant.

The gear ratio is equivalent to the transformer step-up or step-down ratio. A step-up transformer acts analogously to a reduction gear (in which mechanical power is transferred from a small, rapidly rotating gear to a large, slowly rotating gear): it trades current (speed) for voltage (torque), by transferring power from a primary coil to a secondary coil having more turns. A step-down transformer acts analogously to a multiplier gear (in which mechanical power is transferred from a large gear to a small gear): it trades voltage (torque) for current (speed), by transferring power from a primary coil to a secondary coil having fewer turns.

A simple transformer consists of two electrical conductors called the primary winding and the secondary winding. Energy is coupled between the windings by the time-varying magnetic flux that passes through (links) both primary and secondary windings. When the current in a coil is switched on or off or changed, a voltage is induced in a neighboring coil. The effect, called mutual inductance, is an example of electromagnetic induction.

Transformers are adapted to numerous engineering applications and may be classified in many ways:

   By power level (from fraction of a volt-ampere(VA) to over a thousand MVA),

   By application (power supply, impedance matching, circuit isolation),

   By frequency range (power, audio, radio frequency(RF))

   By voltage class (a few volts to about 750 kilovolts)

   By cooling type (air cooled, oil filled, fan cooled, water cooled, etc.)

   By purpose (distribution, rectifier, arc furnace, amplifier output, etc.).

   By ratio of the number of turns in the coils

Exercise 13. Answer the questions.

1.           What devices are described in the texts? 2. What is the dynamo used for? 3. What transformer is used for? 4. What does a dynamo usually include? 5. What does the transformer consist of? 6. What other types of the device do you know?  7. In what ways transformers can be classified?

Exercise 14. Translate the following definitions and memorize the terms.

The parts of a dynamo or related equipment can be expressed in either mechanical terms or electrical terms. Although distinctly separate, these two sets of terminology are frequently used interchangeably or in combinations that include one mechanical term and one electrical term. This causes great confusion when working with compound machines such as a brushless alternator or when conversing with people who are used to working on a machine that is configured differently than the machines that the speaker is used to.

Mechanical
Rotor: The rotating part of an alternator, generator, dynamo or motor.
Stator: The stationary part of an alternator, generator, dynamo or motor.

Electrical
Motors are used for converting different forms of energy into mechanical energy.

Electrical motors are used practically in every branch of industry, transport and agriculture.

Armature: The power-producing component of an alternator, generator, dynamo or motor. The armature can be on either the rotor or the stator.

Field: The magnetic field component of an alternator, generator, dynamo or motor. The field can be on either the rotor or the stator and can be either an electromagnet or a permanent magnet.

Exercise 15. Fill in the blanks with the following words.

conductors, electromagnet, electric energy,

electrical conductor, armature, rotation, current, converted

1. ………..are materials having a low resistance so that current easily passes through them. 2. Dynamo is a common device for converting mechanical energy into …………. . 3. This process depends on the fact that if an ……….. moves across magnetic field, an electrical current flows in the conductor. 4. Usually a dynamo includes an …………., called the field magnet, between the poles of which a suitable conductor, usually in the form, called  the ………….., is rotated. 5. The mechanical energy of the ……….., in the form of a current in the armature, is thus ………… into electric energy.

Exercise 16.  Say a few words about your electrical engineering laboratory. You know that all electrical devices and installations are constructed of a certain number of components. To these components belong electric power sources, resistors, transformers, motors and others. Speak with your groupmates about their types, their use, operation and etc. Have a talk with your groupmates.

Unit 2

Grammar: The Participle

Exercise 1. Participle I/Participle II. Put the Participle in the form suitable for the noun.

A) Participle I                                     B) Participle II

1. (grow) interest                               7. (complicate) explanation

2. (construct) roads                           8. (develop) country

3. (generate) unit                                 9. (obtain) results

4. (run) water                                     10. (use) method

5. (follow) chapter                              11. (destroy) building

6. (insulate) material                           12. (heat)water

Exercise 2. Translate the following word combinations into English paying attention to Participle I/Participle II.

А) 1. методы, улучшающие исследовательскую работу

2. расширяющиеся газы

3. оборудование, установленное в лаборатории

4. проблема, решенная успешно

5. инженер, использующий новые достижения

6. прибор, сконструированный инженером

7. полученные результаты

8.полупроводниковые фотоэлементы, преобразующие солнечную энергию

В) 1. применяя новые методы

2.     получив новые результаты

3.     увеличивая скорость

4.     открыв новый элемент

5. устанавливая новое оборудование

6.     закончив эксперимент

7. соединив два элемента

8.     производя новые приборы

Exercise 3. Translate the following word combinations.

1)           the body lifted, the body possessing, the condition achieved, the system isolating, the energy stored, the force measured, the student attending, the achieved results, the plant producing, the research made, the matter obtained, the substance applied

2)           determining the action, multiplying, having calculated the distance, receiving important information, having installed a new equipment, identifying the element, having determined the substance

Exercise 4. Participle I/Participle II. Choose the form of the Participle in each sentence.

A) Participle I                          B) Participle II

1.     The figures (mentioned, mentioning) in his report were published in the latest scientific journal. 2. The work (performed, performing) showed good results. 3. The type of the particle (emitted, emitting) depended not only on the binding energy. 4. (Being, been) more efficient than human beings computers are used more and more extensively. 5. When (translated, translating), his article on chemistry was sent to the International Chemical Congress. 6. What is he doing? – He is (translating, translated) some article from a foreign journal. 7. The data (obtaining, obtained) formed the basis for the further activity. 8. When (burning, burnt), coal produces heat. 10. The radar (used, using) was of a completely new design. 11. The substance (formed, forming) had all the necessary properties. 12. There are several types of furnaces which are used for metal (melted, melting).

Exercise 5. Translate the following sentences paying attention to the Participle.

1. Creating a new research institute we develop our science. 2. Testing the engine the engineer applied new methods. 3. The questions discussed at our conference are of great significance for the improvement of our scientific research. 4. You have to repair the broken part of that engine. 5. Hydrogen is the lightest element   known.  6. The temperature of the liquid   obtained   remained constant. 7. The explanation of many of the physical phenomena involved presents some difficulties.  8. The channel linking the two seas is being built now. 9. Nuclear fuel is undergoing nuclear fusion. 10. Being a semiconductor, germanium is widely used in transistors.

Exercise 6. Read the following words. What are their prefixes? suffixes? Translate them.

inefficient, superconductor, non-conductor, simplify, waterless, promising, enlargement, interrelation, equality, abundantly, consumption, growth,  advantageous, renewable, unbalanced, inexpensive, conventional, reequip, enlarge

Exercise 7. Translate the following groups of words.

rapid growth in energy consumption; a bigger punch; fuel resource; rapid rise of electrification; major fuel forms; natural gas; nuclear and renewable energy forms;  commercial and industrial enterprises; clean electricity; conventional power plant emissions; nuclear waste from atomic plants; solar, wind and biomass energy forms; wind turbines; world's energy demands; nature and physics of energy forms

Задание для самостоятельной работы 3

Energy

Energy is the lifeblood of our nation. It powers our industries and keeps our economy humming. Our nation's progress has relied on making energy abundantly available to support the growth of new ideas and products. We should understand some of the basics of our energy history and the kinds of fuels that keep things working.

Our century-long, rapid growth in energy consumption has been chronicled by a move to more dense, or concentrated, energy forms. Our early predecessors burned wood. Our great-grandparents used coal, oil, and natural gas. We addressed the need for a rapid increase in energy growth with a bigger punch in a smaller package more energy per pound of fuel resource. This was a good strategy, and it still serves us well although it would be advantageous to decrease our dependence on foreign oil.

The rapid rise of electrification in our homes is a big factor in our energy consumption. This trend began with the great electrical inventions in the late 1800s, which spawned a proliferation of domestic appliances and commercial applications that continues unabated. VCRs, DVDs, personal computers, big screen TVs and microwave ovens are the most recent manifestations of our appetites for kitchen appliances and leisurely pursuits.

Our major fuel forms are coal, oil, natural gas, nuclear and some renewable energy forms, the most notable being hydropower. Electricity is made from these major fuel forms.

Today, we depend more on foreign oil than we did in the 70s, the time of the oil embargos. This oil dependency can be blamed directly on automobiles. Utilities switched from oil to nuclear power decades ago, and today they use natural gas. Many commercial and industrial enterprises also switched to natural gas. Cars consume the majority of imported oil. This makes the move toward more fuel-efficient and hybrid cars of major importance.

There is much interest in renewable energy use. Solar, wind and biomass create clean electricity without conventional power plant emissions or nuclear waste from atomic plants. If you look at the raw numbers, you will quickly realize that the sun showers the Earth with millions of times more energy than we use. But, while the fuel is free, the equipment to utilize it is not. It is the engineer's job to find the point where the desire to use solar energy can be balanced against the costs.

Solar, wind and biomass systems cost more than conventional forms of power generation and they add another dimension to the energy debate. Their usage is similar to using wood as a fuel. Renewable energy systems require large areas to support energy capture. Solar, wind and biomass energy forms over and on the surface of the Earth are rather weak. We must harvest this free, albeit weak, energy over large areas to make it useful. Such areas take the form of solar panels, the swept rotor area of wind turbines or combustible plant species farms. That's the nature and physics of energy forms. We cannot miniaturize the solar panels or concentrate the biomass or wind energy.

We use the energy sources we are willing to tolerate, are comfortable using and willing to pay for. It's a consensus process. What makes our current source so tough to replace is that they have remained quite inexpensive compared with the prices of renewable sources. There is not yet an overwhelming reason or incentive to make great changes in our energy sources. It takes decades to change the overall energy mix. New sources take years to be accepted and become a significant portion of the energy economy.

Our energy use especially electricity will continue to increase. Electricity is too convenient form of energy not to be innovated upon. What happens when population giants like China and India start reaching for higher standards of living, striving to live like Americans do? What then of the world's energy demands? Will that be an incentive for change? It will be interesting to be a part of those decisions!

(By Harry T. Roman

Source: Tech Directions, Apr2006, p19.)

### Exercise 9. Answer the questions.

### 1.           Why energy is called “lifeblood” of our nation? 2. What sources of energy do you know? 3. What are our major fuel forms at present? 4. What sources of energy cannot be replaced when once used up?  5. What is a big factor in our energy consumption?  6. What sources of energy are continually being replaced?  7. What is the most   required form of energy?  8. How many forms of energy do you know? 9. Why is there much interest in renewable energy use?

### Exercise 10. Put down the title corresponding to each of the given descriptions.

### Electric energy, Kinetic energy, Chemical energy,

### Radiant energy, Potential energy, Energy

### 1.  The energy transmitted in the form of radiation.

### 2.  That part of the energy stored within an atom or molecule which can be released by a chemical reaction.

### 3.  The capacity for doing work.

4.  The energy which body possesses by virtue of its position. It is measured by the amount of work the body performs in passing from that position to a standard position in which the energy is considered to be zero.

5.  The energy which body possesses by virtue of its motion.

6.  The energy associated with electric charges and their movements. It is measured in watt seconds (joules) or kilowatts-hours.

### Exercise 11. Define which –ing and -ed forms are parts of predicates.

### 1. The measures discussed are to be used for determining the faults in the conducting wires. 2. The new materials recommended for plant construction were described in the article written by our professor. 3. The velocity of a reactor depends on the specific nature of the substances involved. 4. The transmission system selected for everyday use is based on the combined activity of telecommunication and computers. 5. Water-turbine plants are called hydro turbines. 6. The propeller accelerates the air passing through it thereby transforming engine into useful trust. 7. Gas coolants used to remove heat losses help to increase the current-carrying capacity of the motor’s main parts. 8. Water power is being used to drive a dynamo. 9. The substance affecting a magnetic field was metallic. 10. What problems are being discussed now? 11. At the end of May the engineers will be preparing for their experiments. 12. Today some engineers are working at the problem of developing electric power stations with the use of a thermal-chemical cycle.

### Exercise 12. Fill in the blanks with the following words.

### thermodynamics, heat, energy, conversion,

### degeneration, conservation, constant, mechanical, heat

1. The mechanical theory of heat states that ________ is a form of energy due to the motion or configuration of the molecules of a body. 2. Like _______ energy, heat energy may be of the kinetic or of the potential form. 3. Heat may be transferred either by conduction or by___________. 4. Every natural process is accompanied by thermodynamics _____________. 5. Heat may be converted into work and work into heat and in any such conversion the ratio of the work to heat being constant. 6. __________ deals with the conversion of mechanical energy into thermal energy and   the   reverse   process. 7. To transform energy from its numerous forms into ___________ is a comparatively simple process. 8. Heat is a form of __________ floating from one point to another.

Exercise 13. Translate the following sentences paying attention to Participle I/Participle II.

1. Выполненная работа показала хорошие результаты. 2. Сжигаемый  уголь, производит высокую температуру. 3. Закон, выраженный данной  формулой очень важен. 4. Измерив совершенную работу, инженер продолжил опыт. 5. Испытываемые  материалы различаются по своим свойствам. 6. Падая, тело теряет потенциальную энергию. 7. Тело, обладающее потенциальной энергией, может совершать работу. 8. Работая надусовершенствованием лампы накаливания, Эдисон сделал ряд важных экспериментов.  9. Наблюдая расположение атомов в твердом теле, можно лучше понять его свойства. 10. Данные, упомянутые в его сообщении, были изданы в последнем номере научного журнала. 11. Зная эту величину, вы можете решить это уравнение. 12. По окончании проект должен удовлетворять всем  техническим  требованиям. 13. Результаты, полученные в нашей лаборатории, опубликованы в ежемесячном журнале. 14. Полученные результаты были различными в зависимости от использованного материала.

### Exercise 14. Read the text and find mistakes in each line. If the line is correct put a tick  if not, write down the mistake.

The Law of Conservation energy of means that no energy can _____

be creator or destroyed in any physical effects or transformations.     _____

It has showned however that matter can be converted into energy       _____

of radiation and that therefore the Law of Conservation of                 _____

energy and includes or implies also the conservation of mass. The     _____

fact that mass and energy can be changed one into another has                    _____

given us means to explain the source by the energy radiated by          _____

the Sun for untold billions years of. It is from the melting away         _____

of the Sun’s mass into radiation by degrees.                                               _____

### Exercise 15.  Translate the following supplementary text.